KB-SELEC

SPECIFICATION SHEET

===================

SWITCH MODE POWER SUPPLY

MODEL 		: KPC8-1-110-0050

INPUT

* I/P  				             : 230V AC, 1-Ph 50Hz

* Range                                                    : 180-270V AC

* Fuse Protection                                     : Yes

* I/P MOV                                               : Yes

* Soft Start                                               : Yes

OUTPUT

* Voltage         		                          : 110V DC

* Voltage Range                                       : 110-130V

* Rated Current 	      		: 5A

* Line & Load Regulation		: +/- 1%

* Ripple Max   	       		             : +/- 1% (P-P)

* Protections

: a) Over Load Protection auto recovery type.

b) Short Circuit protection auto recovery type.

c) Over Voltage Protection Latching type.

* Current Limit                                        : Yes

ENVIRONMENTAL

* Operating temperature 0 to 500 C

* Humidity                                               : < 95% Rh

* Enclosure IP 20 Protection

GENERAL

* Isolation                                                : Input to Output – 1.5KV

Input to Earth – 1.5KV

Output to Earth- 500V

LED INDICATIONS  	                          : a) Input OK

b) Output OK

c) Output OVP

MECHANICAL

* Dimension                                             : approx. 235W X 200D X 100H mm

* Cooling type                                          : Fan cooled

* Weight                                                   : approx. 3 Kgs.